Assignment 1 – DEVS Modeling and Simulation

Course: SYSC 5104 / SYSC 4906G

Project: Indoor Humidity Regulation System

Author: Bahareh Pourahmad

Date: February 2026

1. Conceptual Model

1.1 Problem Description

Indoor relative humidity is vital for occupant health and building durability. Indoor humidity continuously drifts toward outdoor levels due to ventilation and leakage. The objective of this project is to model an indoor humidity regulation system that maintains relative humidity (RH) around a 45% reference value using discrete-event state changes rather than continuous equations, making it ideal for the DEVS formalism.

1.2 Modeling Objectives

- Maintain indoor humidity around a reference value of 45%.

- Prevent excessive oscillations using hysteresis-based control.

- Capture environmental leakage effects.

- Provide a hierarchical DEVS structure suitable for Cadmium.

1.3 Hierarchical Model Structure

The system is organized into three levels:

- Top Coupled Model (IndoorHumiditySystem): Integrates all dynamics and the closed-loop mechanism.

- Coupled Model (HumidityPlant): Encapsulates the controller and actuator.

- Atomic Models: Includes IndoorAir, LeakageEstimator, Humistat, and Humidifier.

2. Formal DEVS Specification

Each atomic model follows the classical DEVS formalism: $M = \langle X, S, Y, \delta_{ext}, \delta_{int}, \lambda, ta \rangle$.

2.1 Atomic Model Specifications

1. IndoorAir

- State: $S = \{(RH, phase)\}$, $RH \in [0, 100]$, $phase \in \{passive, update\}$.

- $\delta_{ext}$: Upon receiving leakage or injection, $RH \leftarrow RH + \Delta RH$ and $phase \leftarrow update$.

- $ta$: $ta(passive) = \infty$, $ta(update) = 0$.

2. LeakageEstimator

- Behavior: Every 10 seconds, computes $\Delta RH = 0.1(RH_{out} - RH_{in})$.

- $ta$: 10 seconds.

3. Humistat (Hysteresis Controller)

- Logic: Turn ON if $RH < 43\%$; Turn OFF if $RH > 47\%$.

3. DEVS Graph Specifications

The atomic models were defined using the DEVS Graph Modeler.

- IndoorAir: Transitions from passive to update upon external input, with an immediate internal transition back to passive ($ta=0$).

- Humistat: A two-state machine (ON/OFF) where transitions are governed by the humidity thresholds.

- LeakageEstimator: A single-state model with a looping internal transition every 10 seconds.

4. Implementation and Simulation Results

4.1 Implementation Strategy

The models were implemented in C++ using the Cadmium framework. Each atomic model follows the structure of the Cadmium-Store_Cashier example, utilizing delta_external, delta_internal, output, and time_advance functions.

4.2 Experimental Framework

Validation followed a hierarchical approach:

- Atomic Testing: Verified LeakageEstimator periodic output and Humistat boundary logic.

- Subsystem Testing: Validated the HumidityPlant closed-loop behavior.

- System Testing: Simulated three scenarios: Dry Outdoor (30% RH), Humid Outdoor (70% RH), and Outdoor Step Changes.

4.3 Results Analysis

- Stability: The system stabilizes around the 45% reference.

- Hysteresis: Bounded oscillations were observed between 43% and 47%, preventing Zeno behavior and excessive switching.

- Adaptability: The system successfully adapted to sudden changes in outdoor humidity levels.

5. Checklist and Repository Structure

The project repository on GitHub follows the alternatebitprotocol structure

- /atomics: Source code for all atomic models.

- /coupled: Source code for HumidityPlant.

- /top_model: main.cpp and Makefile.

- /test: Input .ev files and execution scripts.

- /logs: Execution results confirming model correctness.